PC Channel Editor Tool

1-1 Create new data file

[File]  [New] in the menu

1-2 Edit channel data

[File]  [Open] in the menu

1-3 Save data after edit or add channel data

[File]  [Save]

2-1 Add satellite

In the left window, go to [Network] and press right button of mouse. Select [Add Network] then input the information.

2-2 Delete satellite

In the left window, choose a satellite and press right button of mouse on it and select [Del Network] to delete.

2-3 Edit satellite information

In the left window, choose a satellite and press right button of mouse on it and select [Edit].

2-4 Add TP

In the left window, choose a satellite and press right button of mouse on it and select [Add Transport].

2-5 Delete TP

In the left window, choose a TP and press right button of mouse on it and select [Del Transport].

2-6 Edit TP

In the left window, choose a TP and press right button of mouse on it and select [Edit].

2-7 Add channel

In the left window, choose a TP and press right button of mouse on it and select [Add Channel].

2-8 Delete channel

In the right window, choose a channel and press right button of mouse on it and select [Edit] then [Delete].

2-9 Edit channel data

In the right information window, choose a data and double click to edit.

3-1. Transfer satellite data from STB to PC

1. Connect the STB and PC by RS232 cable.

2. Select [Communication]  [Data STB  PC] in the menu.

3. Press [Connect]

4. Power ‘OFF’ and ‘ON’ the STB

3-2 Transfer satellite data from PC to STB

1. Connect the STB and PC by RS232 cable.

2. Select [Communication]  [Data PC  STB] in the menu.

3. Select [Load] and select the file (CDL format).

4. Press [Connect]

5. Power ‘OFF’ and ‘ON’ the STB

3-3 Program update

1. Connect the STB and PC by RS232 cable.

2. Select [Communication]  [Program PC  STB] in the menu.

3. Select [Load] and select the file (SDL format) for update.

4. Press [Connect]

5. Power ‘OFF’ and ‘ON’ the STB

3-4 Transferring Key data from STB to PC

1. Connect the STB and PC by RS232 cable.

2. Select [Communication]  [KD STB  PC] in the menu.

3. Press [Connect]

4. Power ‘OFF’ and ‘ON’ the STB

3-5 Transferring Key data from PC to STB

1. Connect the STB and PC by RS232 cable.

2. Select [Communication]  [KD PC  STB] in the menu.

3. Select [Load] and select the file (KDL format).

3. Press [Connect]

4. Power ‘OFF’ and ‘ON’ the STB

4. All channel list and move channel

1. [Edit]  [Move channel] in the menu